sample_shift_reg.v

module sample_shift_reg (

input  wire       clk,

input  wire       rst_n,

input  wire       valid_in,

input  wire [7:0] sample_in,

output reg  [7:0] s0,

output reg  [7:0] s1,

output reg  [7:0] s2,

output reg  [7:0] s3

);

always @(posedge clk) begin

if (!rst_n) begin          // synchronous active-low reset

s0 <= 8'd0;

s1 <= 8'd0;

s2 <= 8'd0;

s3 <= 8'd0;

end

else if (valid_in) begin   // only shift when data is valid

s3 <= s2;

s2 <= s1;

s1 <= s0;

s0 <= sample_in;

end

// no else needed — FFs hold value automatically

end

endmodule

sum4.v

module sum4 (

input  wire [7:0]  s0,

input  wire [7:0]  s1,

input  wire [7:0]  s2,

input  wire [7:0]  s3,

output wire [9:0]  sum_out

);

wire [8:0] level1_a;

wire [8:0] level1_b;

assign level1_a = s0+s1;

assign level1_b = s2+s3;

assign sum_out  = level1_a + level1_b;

endmodule

smoother.v

module smoother (

input  wire       clk,

input  wire       rst_n,

input  wire       valid_in,

input  wire [7:0] sample_in,

output wire [7:0] filt_out,

output wire       valid_out

);

wire [7:0] s0, s1, s2, s3;

wire [9:0] sum_out;

sample_shift_reg u_shift (

.clk       (clk),

.rst_n     (rst_n),

.valid_in  (valid_in),

.sample_in (sample_in),

.s0        (s0),

.s1        (s1),

.s2        (s2),

.s3        (s3)       // ← no comma here

);

sum4 u_sum (

.s0      (s0),

.s1      (s1),

.s2      (s2),

.s3      (s3),

.sum_out (sum_out)    // ← no comma here

);

assign filt_out  = sum_out[9:2];

assign valid_out = valid_in;

endmodule

Cycle   sample_in   s0    s1    s2    s3    sum    filt_out

1         4                   4     0     0     0      4       1

2         8                   8     4     0     0     12       3

3        12                  12     8     4     0     24       6

4        16                  16    12     8     4     40      10

5        20                  20    16    12     8     56      14

tb_smoother.v

`timescale 1ns/1ps

module tb_smoother;

// 1. declare signals

reg        clk, rst_n, valid_in;

reg  [7:0] sample_in;

wire [7:0] filt_out;

wire       valid_out;

// 2. instantiate DUT

smoother uut (

.clk       (clk),

.rst_n     (rst_n),

.valid_in  (valid_in),

.sample_in (sample_in),

.filt_out  (filt_out),

.valid_out (valid_out)

);

// 3. clock generation

initial clk = 0;

always #5 clk = ~clk;

// 4. expected values

reg [7:0] expected [1:5];

initial begin

// waveform dump — must be first

$dumpfile("smoother.vcd");

$dumpvars(0, tb_smoother);

// load golden reference

expected[1] = 1;

expected[2] = 3;

expected[3] = 6;

expected[4] = 10;

expected[5] = 14;

// 5. apply reset

rst_n     = 0;

valid_in  = 0;

sample_in = 0;

@(posedge clk); #1;

rst_n = 1;

// 6. stimulus + checking

// Cycle 1

valid_in  = 1;

sample_in = 4;

@(posedge clk); #1;

if (filt_out !== expected[1])

$display("MISMATCH cycle 1: got %0d, expected %0d", filt_out, expected[1]);

else

$display("PASS cycle 1: filt_out = %0d", filt_out);

// Cycle 2

sample_in = 8;

@(posedge clk); #1;

if (filt_out !== expected[2])

$display("MISMATCH cycle 2: got %0d, expected %0d", filt_out, expected[2]);

else

$display("PASS cycle 2: filt_out = %0d", filt_out);

// Cycle 3

sample_in = 12;

@(posedge clk); #1;

if (filt_out !== expected[3])

$display("MISMATCH cycle 3: got %0d, expected %0d", filt_out, expected[3]);

else

$display("PASS cycle 3: filt_out = %0d", filt_out);

// Cycle 4

sample_in = 16;

@(posedge clk); #1;

if (filt_out !== expected[4])

$display("MISMATCH cycle 4: got %0d, expected %0d", filt_out, expected[4]);

else

$display("PASS cycle 4: filt_out = %0d", filt_out);

// Cycle 5

sample_in = 20;

@(posedge clk); #1;

if (filt_out !== expected[5])

$display("MISMATCH cycle 5: got %0d, expected %0d", filt_out, expected[5]);

else

$display("PASS cycle 5: filt_out = %0d", filt_out);

$finish;

end

endmodule